# Analysis of Initial Motion and Context Data (Test Run)

This document provides a comprehensive analysis of the initial data captured by the ESP32 multi-sensor device. The findings confirm that the decoupled logging architecture has produced a high-quality, highly consistent dataset that is perfectly structured for both traditional Machine Learning (ML) and Deep Learning (DL) applications.

## 1. Executive Summary: Why This Dataset is Excellent

The data is of research-grade quality due to its synchronised, multi-modal structure and built-in quality control.

| Metric | Finding | Significance |
| --- | --- | --- |
| Data Integrity | 100 % complete data in both files. | No pre-processing required for missing values. |
| Sensor Rate | approx. 50 Hz IMU/Magnetometer. | Sufficiently high frequency to capture subtle human gait patterns. |
| Label Quality | Excellent Consistency (Median sample step of 49) | Perfect alignment of high-frequency sensor data with the low-frequency 1 Hz GPS label. |
| Quality Control | Minimum Satellites (5) confirmed. | The GPS fix-gate logic successfully prevented the logging of unreliable positional data. |

## 2. Data Structure and Quality Control

### Sensor Data (sensor.csv)

This file contains 19,864 samples of high-frequency time-series data, recorded over 6.62 minutes. It provides the raw inputs for a movement model:

- Inertial Measurement Unit (IMU): AccX/Y/Z and GyroX/Y/Z.

- Magnetometer: MagX/Y/Z.

### GPS Label Data (gps_label.csv)

This file contains 420 entries of low-frequency, contextual ground-truth labels. Critically, it contains the StartSampleID which acts as the synchronisation key to link the GPS data with the continuous sensor stream.

### Consistency Check: Decoupled Sampling Success

The core technical success of the system lies in the stable coupling of the 50 Hz sensor data with the 1 Hz GPS data. The difference between consecutive StartSampleID values should be exactly 50 (representing 1 second times 50 samples/second).

The statistics below confirm near-perfect, stable timing:

| Statistic | GPS StartSampleID Step | Interpretation |
| --- | --- | --- |
| Count | 420 | Total number of 1-second windows captured. |
| Median | 49 | The most common number of sensor samples between GPS fixes. |
| Max | 49 | Maximum samples recorded in a window. |
| Mean | 47.29 | The average number of samples per window. |

This high consistency means the data requires minimal preparation for Windowing and Label Alignment, which is the fundamental process for creating the final ML dataset.

## 3. Deep Learning (DL) Suitability

This dataset is exceptionally well-suited for advanced DL architectures, specifically Convolutional Neural Networks (CNNs) and Long Short-Term Memory (LSTMs).

### A. High-Frequency Input for CNN/LSTM

The 50 Hz sampling rate is ideal for DL models. Instead of relying on hand-engineered statistical features (mean, variance), the models can be fed the raw approx. 50 samples within each 1-second window.

- CNNs can learn subtle local patterns in the signal (e.g., the precise impact signature of a footstep).

- LSTMs can model the temporal sequence of motion (e.g., the evolution of acceleration during a step cycle).

### B. High-Fidelity Labelling

The GPS data provides a rich, automatically derived ground-truth label for every 1-second window of motion.

- Regression Labels: The model can be trained to predict the continuous value of Speed kph from the IMU data alone.

- Classification Labels: Derived labels like Walking, Jogging, or Uphill (derived from Alt change) can be applied directly to the time-synchronised sensor windows.

### C. Multi-Modal Context

The inclusion of Magnetometer data adds environmental context, allowing the model to distinguish between geometrically similar movements in different environments (e.g., distinguishing smooth open-field motion from jittery urban movement).

## 4. Visual Inspection of Activity

The plots generated during the initial analysis clearly demonstrate real-world, dynamic activity:

### Accelerometer Data (Example Window)

The raw accelerometer plot exhibits clear, high-frequency oscillations consistent with cyclical human motion, such as walking or jogging, providing the characteristic patterns needed for accurate Human Activity Recognition (HAR).

### GPS Context (Speed and Altitude)

The GPS plot provides the ground-truth context:

- Speed: Shows activity consistently in the 4 to 8 km/h range, confirming a period of sustained movement (e.g., walking/jogging).

- Altitude: Reveals a steady upward trend, meaning the entire period can be labelled as Uphill, adding a critical context label to the dataset.